Script CS-Demo

How to install

Café Station 1™

Attach the two John Guest 3/4" – 3/8”

threaded fitting to their counter

parts on control box.

Press on the union connector

3/8” to the outlet fitting.

Add stop plug to the fitting

with the drop symbol.

Twist tightly to secure fit.

Attach 2 screws, suited for your wall,

with cc 90 mm levelled.

Always hang the control box upright

with outlet upwards.

Make sure control lights

are visible at all times.

Attach shut off valve to purifier tube

before attaching the tube to the right inlet valve.

Plug in stem elbow 3/8” to outlet union connector

Attach silicon tube with the shortest

length to the nozzle on the bottom of the pump.

Ligthly press the duck bill valve

so it opens up and release.

Place the black rubber o-ring in the

5 L mineral solution adapter, press down to fit,

and then attach the adapter to the jug of minerals.

Place the Dosing Pump on the adapter,

make sure the silicon tube hangs freely

into the minerals, and screw to fasten.

Thread silicon tube coming out from the pump

head over the hose stem 1/4” – 3/16” all the way down the shaft.

Thread silicon tube coming out from the pump

head over the hose stem 1/4” – 3/16” all the way down the shaft.

Insert stem reducer into check valve 3/8” .

Make sure arrow is pointing away from stem reducer.

Attach 40 mm John Guest tube 3/8” to check valve 3/8”.

Insert check valve tube into one of the parallel openings in the two way divider 3/8”.

Attach John Guest 3/8” blue tube to connected

stem elbow 3/8” outlet on control box.

Cut blue tube from control box according

to distance of pressure less tank and the control box

and attach blue tube to other parallel opening in two way divider.

Insert blue John Guest tube to last opening in the two way divider.

Cut to adjust suitable length before attaching it to tank inlet valve on the right side.

The valve without any connections inside tank.

If more distance is needed between control box and

pressure less tank, or you need to adjust angels for fittings,

use a longer John Guest blue tube and/or the stem

elbow 3/8" to connect to the pressure less tank.

If extra length is needed between

mineral jug and pressure less tank,

attach the silicon tube connector

to pump tube opening and the extra

silicon tube to the connector

and proceed with installation.

Attach the cable for float switch to the

connection on the pressure less tank. Tighten ring securely.

Attach the other end to the connection

on the control box with 4 small holes.

Tighten ring securely.

Remove protecting caps from inlet and outlets on pump with pressure tank.

Attach 1 adapter 1/2" to outlet.

Depending on placing of pump, plug in 1 elbow fitting or 1 straight

fitting 1/2"BSP in inlet and attach 1 adapter 1/2" on elbow fitting.

Decide where to place pump with

pressure tank and attach with the

On/Off button always facing upwards

Connect the stem elbow 3/8” on a piece of

John Guest tubing and connect to the last opening on the pressure less tank.

Cut to fit connection on pump.

Connect another stem elbow 3/8” on the end

of the John Guest blue tube and connect to adapter on pump inlet valve.

Note: elbow connections aren’t necessary

if they are not needed due to different angles of tubes in the installation.

Connect John Guest tube to pump outlet adapter.

We suggest mounting the tube to the wall for a more

sturdy installation. Choose mounting

according to wall material.

Cut John Guest tube at your choice of length to

fit on the valve 3/8” and connect the tube

on the other end of the valve.

Attach the equal tee 3/8” at the end point.

Connect one John Guest tube

per outlet from equal

tee connection.

One for a faucet and one for

espresso machine.

Connect John Guest tube to espresso machine

either directly or to the espresso machines own external inlet tubes.

Connect power cords to pump head, control box and

pump with pressure tank.

Connect the pump by pressing the

bottom button on the control box

until the big blue light on top of the

pump starts to blink and the button

you pressed also blinks.

Press the light on the pump

head once for pairing.

Prime the pump head by pressing

the light and holding it down until

it is green, keep pressing until you

see the silicon tube getting

completely saturated with

minerals the whole way.

For this next step, please locate the power source to the purifier that supplies the

system with water in case you need to cut the water supply.

Choose the right mode by pulling out

the power cord to the control box.

Hold down the top button

reconnect the power cable.

Release the button.

Count the number of blue light blinks

until it skips one blink.

If it blinks 3 times and then skips one,

it is in Mode 3.

(Note: There are seven modes total.)

Press the

blinking button until it only blinks

once with a skip between.

That is Mode 1 for espresso machine.

Confirm mode by pressing the

bottom button. The top button

will turn on a purple light

and the water starts to flow.

When the tank is full, the float switch will cut the flow

and the purple light will turn off and system is ready for use.

Important:

If the water keeps filling up

past ≈ 80% and approaches

the full capacity of the

pressure less tank,

cut the power to the purifier as

mentioned in the beginning

of this clip.

Open the faucet to drain about half

the amount of water in the

pressure less tank.

Go through the installation again

to check for misses, especially the

black power connections

to the floatswitch.

Retest the installation by giving

power to the purifier again to

activate the water flow.

If the float switch still isn’t

stopping the flow upon filling

the tank cut the power

to the purifier and call your

local support for assistance.

Café Station 1™

will now provide the best water for coffee.