How to factory reset the pump and control box.

- Use the app and choose “Unpair dosing pump” in the menu.

- Close the app. (It is important that it is closed and not only minimized or it will keep the Bluetooth interface open)

- Perform a factory reset of the controlbox.

  - Remove power from the controlbox.

  - Press and hold both buttons on the controlbox while applying power to the contolbox.

  - Keep both buttons pressed. (Both orange LEDs shall be lit)

  - When both lights go out the reset is finished.

- Perform a factory reset of the Dosing pump.

  - Remove power from the dosing pump

  - Press and hold the button on the top of the pump head.

  - Keep it pressed until the orange light goes out.

  - When the light goes out, the reset is finished

- Put the contolbox in mode 1

- Pair the dosing pump

- Start the app

This is a suggestion to solve issues when the pump isn’t dosing properly even though the lights are on on both systems. The factory reset is a great first line support suggestion to technicians in the field and on site trying to fix an issue.

This is successful if when the system is producing RO water after the RO machine and it gets mineralised while passing through the control box to a faucet or into a pressureless tank.